NAME:ARUN.T

REGNO:1912260643

EXPERIMENT NO:2

MATLAB Simulation of Cellular Coverage, Cell Splitting and Handoff in a Multi-Cell  Network

MATLAB Program :Coverage, Cell Splitting and Handoff

% Experiment 2: simplified hexagonal coverage and handoff

clear; clc; close all;

% Baseline assumptions

serviceAreaKm2 = 5;

radiiKm = [1.0 0.5];

usersPerCell = 30;

hexArea = @(R) (3*sqrt(3)/2) .* R.^2;

cellAreaKm2 = hexArea(radiiKm);

cellCount = ceil(serviceAreaKm2 ./ cellAreaKm2);

estimatedUsers = cellCount .* usersPerCell;

capacityTable = table(radiiKm(:), cellAreaKm2(:), cellCount(:), ...

repmat(usersPerCell,numel(radiiKm),1), estimatedUsers(:), ...

'VariableNames', {'Radius_km','CellArea_km2','EstimatedCells', ...

'UsersPerCell','IllustrativeUsers'});

disp(capacityTable);

% Plot a 7-cell cluster: central cell plus six neighbors

R = radiiKm(1);

centers = [0 0; 1.5*R sqrt(3)*R/2; 1.5*R -sqrt(3)*R/2; ...

0 sqrt(3)*R; 0 -sqrt(3)*R; -1.5*R sqrt(3)*R/2; ...

-1.5*R -sqrt(3)*R/2];

theta = (0:6)*pi/3; % flat-top hexagon vertices

figure; hold on; axis equal; grid on;

for k = 1:size(centers,1)

plot(centers(k,1)+R*cos(theta), centers(k,2)+R*sin(theta), 'k-');

text(centers(k,1),centers(k,2),sprintf('Cell %d',k), ...

'HorizontalAlignment','center');

end

xlabel('x (km)'); ylabel('y (km)');

title('Idealized Seven-Cell Hexagonal Cluster');hold off;

% Mobile moves across the boundary between centers at x=0 and x=1.5R

mobileX = linspace(-0.2*R,1.7*R,300);

mobileY = zeros(size(mobileX));

d1 = hypot(mobileX, mobileY);

d2 = hypot(mobileX-1.5*R, mobileY);

servingCell = ones(size(mobileX));

servingCell(d2 < d1) = 2;

handoffIndex = find(diff(servingCell) ~= 0,1,'first') + 1;

figure; hold on; grid on;

plot(mobileX,d1,'LineWidth',1.5);

plot(mobileX,d2,'LineWidth',1.5);

if ~isempty(handoffIndex)

xline(mobileX(handoffIndex),'--r','Handoff decision');

end

xlabel('Mobile x-position (km)'); ylabel('Distance to cell center (km)');

legend('Cell 1','Cell 2','Location','best');

title('Nearest-Center Handoff Approximation');

hold off;

figure; stairs(mobileX,servingCell,'LineWidth',1.5); grid on;

xlabel('Mobile x-position (km)'); ylabel('Serving-cell ID');

title('Serving Cell Along the Mobile Path');

if ~isempty(handoffIndex)

fprintf('Geometric handoff at x = %.3f km\n',mobileX(handoffIndex));

end